#### 1. Write a Program in C/ C++ for error detecting code using CRC-CCITT (16 bit)

Psedocode:

Start

Input generator polynomial (gen)

Input message bits (msg)

genlen = length(gen)

k = genlen - 1

Append k zeros to msg → a = msg + "0" * k

For i = 0 to length(msg) - 1:

If a[i] == '1' then

For j = 0 to k:

a[i + j] = XOR(a[i + j], gen[j])

EndIf

EndFor

Remainder = last k bits of a

Transmitted = msg + Remainder

Print Remainder and Transmitted

Input received message (s)

For i = 0 to length(s) - 1:

If s[i] == '1' then

For j = 0 to k:

s[i + j] = XOR(s[i + j], gen[j])

EndIf

EndFor

Remainder = last k bits of s

If Remainder == all zeros:

Print "Received message is error-free"

Else

Print "Error detected"

EndIf

Stop

Code:

#include <stdio.h>

#include <string.h>

#include <stdlib.h>

int main() {

char rem[50], a[50], s[50], c, msg[50], gen[30];

int i, j, k, n, t, flag = 0;

int genlen;

printf("Enter the generator polynomial: ");

gets(gen);  // Example: 10001000000100001 (for CRC-CCITT)

printf("Generator polynomial is CRC-CCITT: %s\n", gen);

genlen = strlen(gen);

k = genlen - 1;

printf("Enter the message: ");

n = 0;

while ((c = getchar()) != '\n') {

msg[n++] = c;

}

msg[n] = '\0';

// Append k zeros to message

for (i = 0; i < n; i++)

a[i] = msg[i];

for (i = 0; i < k; i++)

a[n + i] = '0';

a[n + k] = '\0';

printf("\nMessage polynomial appended with zeros:\n");

puts(a);

// Division process (CRC generation)

for (i = 0; i < n; i++) {

if (a[i] == '1') {

t = i;

for (j = 0; j <= k; j++) {

a[t] = (a[t] == gen[j]) ? '0' : '1';

t++;

}

}

}

// Remainder is checksum

for (i = 0; i < k; i++)

rem[i] = a[n + i];

rem[k] = '\0';

printf("\nChecksum (Remainder): ");

puts(rem);

// Append checksum to original message

for (i = 0; i < n; i++)

a[i] = msg[i];

for (i = 0; i < k; i++)

a[n + i] = rem[i];

a[n + k] = '\0';

printf("\nTransmitted message (message + checksum): ");

puts(a);

// Receiver side

printf("\nEnter the received message: ");

n = 0;

while ((c = getchar()) != '\n') {

s[n++] = c;

}

s[n] = '\0';

// Division process at receiver side

for (i = 0; i < n; i++) {

if (s[i] == '1') {

t = i;

for (j = 0; j <= k; j++, t++) {

s[t] = (s[t] == gen[j]) ? '0' : '1';

}

}

}

// Extract remainder

for (i = 0; i < k; i++)

rem[i] = s[n + i];

rem[k] = '\0';

// Check for errors

flag = 0;

for (i = 0; i < k; i++) {

if (rem[i] == '1') {

flag = 1;

break;

}

}

if (flag == 0)

printf("\n Received message is error-free.\n");

else

printf("\n Error detected in the received message.\n");

return 0;

}

Output: